Radiated Immunity

Tests according to

Standard: TS 151.010-1

Conclusion：

1         Total

0        Pass

1        Failed

The test results relate only to devices specified in this document

# Contents

1.	Contents	2

2.	Introduction	3

3.	test equipment	3

4.	measurements summary	3

6.   Audio BREAKTHROUGH from phone	4

change history	12

# Introduction

This procedure describes how to perform the EMC test for radiated immunity according EN 61000-4-3 standard. The purpose of this test is to ensure the immunity of the equipment in a radio-frequency field. The test is divided into three parts, one for unintentional transmission when the phone is in idle mode, one for audio breakthrough when a call is active and one when a datacall is active.

# test equipment

| Name | Device Type | Manufacturer | Calibration Date |
| --- | --- | --- | --- |
| EMS Test System | TS 9982 | ROHDE&SCHWARZ | 22.02.07 |

# measurements summary

Audio Breakthrough

| Go to test No. | V/m | Measured from | HWID | Horizontal |  |  |  |  |  |  |  |  |  |  |  | Vertical |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 0° |  |  | 90° |  |  | 180° |  |  | 270° |  |  | 0° |  |  | 90° |  |  | 180° |  |  | 270° |  |  |
|  |  |  |  | UL | DL | RX QUAL | UL | DL | RX QUAL | UL | DL | RX QUAL | UL | DL | RX QUAL | UL | DL | RX QUAL | UL | DL | RX QUAL | UL | DL | RX QUAL | UL | DL | RX QUAL |
| 6.1 | 6 | Phone |  | P | P | P | P | P | P | P | P | P | P | P | P | P | F | P | P | P | P | P | P | P | P | F | P |

| DUT | TD | Build | IMEI | HW | SW | PSN |
| --- | --- | --- | --- | --- | --- | --- |
| 18# |  |  | 004400003501119 |  |  |  |

| Accessory | Type | Serial # | HW | SW | MV |  | Manufacturer | DUT No. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| charger | NTPCA-053065C | SN:TPI681402478 |  |  |  |  |  |  |
| Battery | HBC80S | SN:HGY693002941 |  |  |  |  |  |  |
| Headset |  |  |  |  |  |  |  |  |

# 6.   Audio BREAKTHROUGH from phone

| Test 1 | Radiated Immunity |
| --- | --- |
| Operators | Chen Wenjun |
| DUT 18# | IMEI: 004400003501119 |
| Operating mode | GSM900 Call mode (power level 5, traffic channel 65) |
| Charger | NTPCA-053065C |
| Battery | HBC80S |
| Headset |  |
| Date | 04.11.2007 |
| Temp. / Humidity | 23°C / 40 % |
| Comment | Measured from phone |
| Result (*) | F |

(*)  P: passed      F: failed *: partly done

6AudGSM H-0d

6AudGSM H-90d

6AudGSM H-180d

6AudGSM H-270d

6AudGSM V-0d

6AudGSM V-90d

6AudGSM V-180d

6AudGSM V-270d